| To |  | From |
| --- | --- | --- |
| Eduardo Calvo |  | Amit Thomas, Jerry Shadewald, Eric Youngblom, Darrin Willer |
|  |  | Date |
|  |  | 10/10/2012 |
|  |  | Subject |
|  |  | Border Highway West – Preliminary Toll Traffic and Revenue Study |
|  |  | HNTB Job Number |
|  |  | 42085 |
| Technical Memorandum |  |  |

This technical memorandum describes the methodology, assumptions and forecasts based on a preliminary toll sensitivity analysis for the Border Highway West (BHW) toll road in the El Paso region of Texas. In addition to toll sensitivity analysis, this study presents the forecasted toll traffic and revenue for Border Highway West from the opening year 2015 to future year 2055. These traffic and revenue forecasts are preliminary and they are not intended to be used directly in support of project financing.

Project Background

Border Highway West is a proposed 4-lane controlled-access toll facility in El Paso, Texas. The project limits extend from Racetrack Drive near Doniphan Road and New Mexico (NM) 273 on the west to United States Highway (US) 54 east of downtown, a total project length of approximately 9 miles. All existing roadways would remain non-tolled. The project is also listed in the Mission 2035 Metropolitan Transportation Plan (from east of Park Street to the Schuster Extension/Spur 1966). For purposes of this study, BHW is proposed to extend further west to I-10 just south of the collector-distributor (CD) project limits on US 85.

Project Study Area

Figure 1 shows the location of the proposed Border Highway West toll road in the El Paso area in Texas. As shown in Figure 1, Border Highway West is a proposed tolled facility that will extend from Loop 375 at Santa Fe in downtown El Paso to Racetrack Drive near Doniphan Road and New Mexico (NM) 273 on the west (I-10 just south of the collector-distributor (CD) project limits on US 85). Some segments of the toll road are in existence, but other segments are proposed as new roadway sections. The major east-west competing facilities in the corridor include US-85, I-10, and Texas Highway 20, and a few additional competing arterial roadways like Missouri,  Wyoming, Montana, Arizona Avenue, Yandell Drive, Texas and San Antonio Avenues, in the southern section of the corridor.

For the purposes of toll revenue analysis, only the portion west of the Santa Fe bridge is tolled. The portion of BHW east of Santa Fe to US 54 (including the Coles connector) is assumed to be toll-free.

Figure 1: Study Area showing Project Location

Figure 2: Toll Schematic showing Proposed Toll Plaza Locations

Ramp Gantry

Mainline Gantry

Toll Concept

Figure 2 shows the location of the mainline and ramp toll gantries on the proposed Border Highway West toll road. In the northern section (north of Executive Center), there is one mainline plaza and two ramp plazas on the ramps to and from Doniphan Drive. In the middle section (between Executive Center and Spur 1966), there are four ramp toll plazas – northbound on ramp at Spur 1966, southbound off ramp at Spur 1966, northbound off ramp at Executive Center, southbound on ramp at Executive Center. In the southern section, there is a mainline toll plaza south of Spur 1966.

Travel Demand Model

The travel demand model used for this study is the enhanced Mission model. The Transportation Planning and Programming (TPP) Division of TxDOT developed this model in collaboration with the El Paso Municipal Planning Organization (MPO). Trip generation, trip distribution and mode choice components were run by TPP and the resulting trip tables and network related pieces needed for traffic assignment were provided to HNTB.

HNTB added network detail for No-Build and Build alternatives and developed forecasts for various scenarios including forecasts for the  Draft Environmental Impact Study (DEIS).

Network Modifications

Several modifications to the existing and future transportation system have been identified as part of the 2035 design year conditions for BHW.  A detailed description of all the network changes is available in the traffic section of the Environment Impact Study (EIS). These changes include the following additions and modifications to the existing roadway system:

- Direct connections between US 85 and Sunland Park at I-10, along with extending the CD north through Resler Drive ramps to Mesa Street.

- Inclusion of the Desert Pass development between Resler Drive and Mesa Street, east of I-10.  The development will have primary access at Resler Drive, but a right-in, right-out on the northbound CD facility is anticipated.

- The existing standard diamond interchange between I-10 and Executive Center will be modified to a split diamond configuration, connecting to the proposed extension of Mesa Park to cross over I-10.

- Inclusion of Spur 1966, connecting US 85 with Schuster Avenue near the UTEP campus, which removes the existing Yandell bridge over I-10, and expansion of SunBowl Drive to four continuous lanes between Schuster Avenue and Mesa Street.

- Access to downtown El Paso would be limited to right-in, right-out access, with full access provided at Spur 1966 west of downtown and via new direct connectors to Paisano near Coles Street east of downtown.

- Inclusion of high-speed ramp connections between I-10 and Loop 375 near US 54/I-110.

Sub-area Model

TPP recommended a two percent growth rate in the BHW, I-10, US 85, and Mesa corridor.  The growth rate for these corridors resulting from the enhanced Mission model was lower than this recommended rate.  Since the enhanced Mission model did not generate adequate traffic to replicate year 2035 forecasted volumes using the two percent growth rate, HNTB developed a sub-area model and used a Fratar process to achieve this traffic growth rate at key screen line locations in the BHW, I-10, US 85 and Mesa corridors including at the externals on the east and west ends of the sub-area.

The goal of the subarea model is to achieve the desired growth rate within the sub-area section of the entire model.  This geographic extent should allow for traffic diversions of motorists who want to divert from the BHW facility to avoid paying tolls.

The subarea model was developed to test the traffic and revenue toll sensitivity of the proposed BHW toll facility.  The roadway network for this subarea model includes US 85, I-10, and Mesa between Mesa/I-10 in the northwest and Coles direct connectors/Loop 375 on the southeast end of the project.

Figure 3 and Figure 4 show the scope of the subarea model compared to the enhanced Mission model.

Trip Table Factoring

The subarea model uses a pattern OD matrix from the full enhanced Mission model and applies Fratar factoring at the external entry points (of the subarea model) to replicate the two percent traffic forecasts.

External stations on I-10 (northwest and southeast end), I-10 CD north of Mesa, and Loop 375 east of Coles direct connectors had target volumes equal to values found on the year 2015  and 2035 forecast line diagrams.  All other centroids and external stations were factored globally from Mission model values to meet screenline forecasted volumes.

Figure 3: Mission Model network with outlined subarea for BHW analysis

Figure 4: Subarea network

Figure 5 through Figure 6 show the location of the screenlines where the subarea model results were compared to the previously forecasted traffic line diagrams in the draft EIS document.  Table 1 and Table 2 show (for future years 2015 and 2035) the comparison of traffic volumes from the subarea model at key screenline locations in the corridor compared to forecasted line diagrams (used for the EIS document traffic forecasts) the target growth rate of 2 percent per year recommended by TPP.

This growth rate of 2 percent per year resulted in significantly higher volumes for the opening year 2015 and 2035 than the corridor volumes from the enhanced Mission model. Since the growth rates differ considerably, two separate sets of trip tables representing these scenarios were used for toll analysis. The trip tables corresponding to traffic volume forecasts with a corridor growth rate of 2 percent per year will be referred to in this study as the high growth scenario, while the enhanced Mission model forecasts are considered as the low growth scenario.

A third set of trip tables, representing a medium growth scenario, was developed by calculating the average between the low and high growth scenario trip tables.

Figure 5: Screenline south of Executive Drive

Figure 6: Screenline north of Executive Drive

Figure 7: Screenline north of Sunland

Figure 8: Screenline North East of Cotton

Table 1: Comparison of screenline volumes for the 2035 Forecasts and Subarea Model Volumes

| Location | Forecast Volumes | Subarea Volumes | Ratio |
| --- | --- | --- | --- |
| South of Executive | 322,000 | 320,383 | 0.99 |
| North of Executive | 329,600 | 324,993 | 0.99 |
| North of Sunland | 277,000 | 269,624 | 0.97 |
| East of Cotton | 419,300 | 425,847 | 1.02 |

Table 2: Comparison of screenline volumes for the 2015 Forecasts and Subarea Model Volumes

| Location | Forecast Volumes | Subarea Volumes | Ratio |
| --- | --- | --- | --- |
| South of Executive | 206,900 | 214,000 | 1.03 |
| North of Executive | 220,300 | 218,700 | 0.99 |
| North of Sunland | 186,800 | 187,200 | 1.00 |
| East of Cotton | 301,500 | 294,900 | 0.98 |

Corridor Growth Scenarios

Three corridor growth scenarios were assumed in the corridor to analyze toll sensitivity:

- high growth scenario, which uses the trip tables based on the 2 percent per year growth rate across screenlines specified by TPP

- low growth scenario, which uses the trip tables directly from the enhanced Mission model

- medium growth scenario, which uses the average of the trip tables from the low and high growth scenarios. HNTB developed toll sensitivity curves to determine the optimal toll rate per mile for revenue maximization based on this medium growth scenario.

The medium growth scenario uses the average of the trip tables between the low and high growth scenario, and this represents a conservative growth scenario for the corridor. While toll sensitivity was analyzed for all three growth scenarios, the focus of this study is the medium growth scenario.

Traffic and revenue toll sensitivity curves are shown only for the medium growth scenario. However, the detailed transactions and revenue for each section for all scenarios at the optimal toll rates are shown in this memo.

Truck Toll Rates

Trucks were on average assumed to have an average toll rate per mile of three times that of the average toll rate per mile of autos.

Electronic and Video Tolling

Both electronic toll collection (ETC) and video tolling options are assumed to be available to potential users of BHW. For this preliminary toll study, a hybrid of the ETC and video toll rates are assumed.

In the opening year 2015, the ETC percentage was assumed to be 40 percent with the remaining 60 percent of users being charged at the video rate. The video rate is assumed to be 50 percent higher than the ETC rate.

By 2035, 60 percent of all users are assumed to be ETC users. In this study, the nominal toll rate refers to the ETC auto toll rate per mile.

Value of Time Assumptions

Using the MPO demographics documentation from Alliance Transportation Group (ATG) and weighting using the TAZ demographic data, an average value of time (VOT) that is weighted by the households in each TAZ was developed for the toll model runs.

Assuming 2,080 working hours a year and a value of time equaling half the median household income, a value of time (VOT) of $8.49 (2009 dollars) was calculated. This value inflated for future model years by two percent per year annually was used in the toll sensitivity analysis. This calculated VOT compares well to a VOT of $8.40 used in 2008 in a previous traffic and revenue forecast study by CDM Smith.

Toll Sensitivity Testing

HNTB used the subarea travel demand model to test toll sensitivity for three growth scenarios. Details of the toll methodology used for testing toll sensitivity are detailed in the HNTB technical memorandum entitled “Enhanced Mission Model Tolling Methodology” from January 2012 also in the BHW Forecast Methodology memo.

HNTB tested various toll rates for each scenario to identify the maximum revenue potential by time period. The average toll rates per mile tested for the medium growth scenario ranged from $0.05 to $0.25 per mile (in year 2015 dollars) for future year 2015 and $0.25 to $0.70 per mile (in year 2035 dollars) range for future year 2035.

Figure 9 and Figure 10 show the average daily revenue toll sensitivity curves for BHW in the medium growth scenario for the opening year 2015 and the future year 2035. The toll sensitivity curves indicate how the forecasted daily revenue varies based on changes in the average toll rate per mile. As the curves indicate, the toll sensitivity is different in the opening year 2015 as compared to the future year 2035.

The optimal toll rate was determined to be $0.10 per mile for the year 2015 and $0.60 per mile on average for future year 2035.

Figure 9: Average Daily Revenue Toll Sensitivity Curve for Year 2015

Figure 10: Average Daily Revenue Toll Sensitivity Curve for Year 2035

Estimated Average Weekday Traffic Volumes

Figure 11 and Figure 12 show the estimated average weekday traffic for mainlines and ramps on various sections of BHW for 2015 and 2035 in the medium growth scenario at the optimal toll rates for each year.

Figure 11: Estimated Average Weekday Traffic Volumes for Opening Year 2015 at Optimal Toll Rate

Figure 12: Estimated Average Weekday Traffic Volumes for Future Year 2035 at Optimal Toll Rate

Preliminary Annual Transactions and Revenue

Opening Year

The assumed opening year for the BHW toll road is 2015. This modeling effort did not assume a ramp-up period. HNTB recommends a 3-6 year ramp up period during which the full potential toll traffic may not use the facility because motorists are becoming aware of the facility, the toll rates and payment options available.

Assumptions

The daily and annual toll transactions and revenue presented here are based on optimal toll rates assuming variable tolling for AM, midday, PM, and night time periods. With variable tolling, the same toll rate does not necessarily apply to every time period. Further, for 2015 and 2035, the optimal rate per mile is different for each scenario (high, low and medium growth) and for each section of BHW (north, middle, and south). After the year 2035, the transactions and traffic were assumed to grow at an average rate of 2 percent per year annually.

Typically, the same optimal rate was chosen for both the AM and PM time periods. The optimal rates per mile for the midday were usually lower than the AM and PM period optimal rates. The night period generated very little or no revenue in many cases even at low toll rates because I-10 is uncongested during this period.  Since the AM and PM time periods account for a majority of the anticipated traffic on the facility, the optimal rates chosen for the AM/PM time period will be referred to as the optimal or nominal rate in this study.

Average Daily Transactions and Revenue

The traffic and revenue (in future year dollars) forecasts are broken into the three sections:

- BHW North section from I-10 to Executive Center (approximately 1.3 miles)

- BHW Middle section from Executive Center to Spur 1966 (approximately 2.2 miles)

- BHW South section from Spur 1966 to downtown El Paso (approximately 1.6 miles)

Table 3 and Table 4 show the preliminary average daily tolled VMT, daily transactions and daily revenue using the optimal rates for the medium growth scenario (in future year dollars). The optimal toll rates were $0.10 (2015 dollars) per mile for all sections of BHW in 2015 and they increased to $0.40, $0.50 and $0.40 per mile for the north, middle and south sections of BHW for year 2035 (2035 dollars). The corresponding gross annual revenue (for medium, high and low growth scenarios) is presented in Table 7. The revenues shown are in future year dollars.

Table 3: Daily Vehicle Miles Traveled for Medium Growth Scenario

| Year | BHW North | BHW Middle | BHW South | Total |
| --- | --- | --- | --- | --- |
| 2015 | 15,900 | 11,700 | 22,500 | 50,100 |
| 2016 | 16,830 | 13,295 | 22,790 | 52,915 |
| 2017 | 17,760 | 14,890 | 23,080 | 55,730 |
| 2018 | 18,690 | 16,485 | 23,370 | 58,545 |
| 2019 | 19,620 | 18,080 | 23,660 | 61,360 |
| 2020 | 20,550 | 19,675 | 23,950 | 64,175 |
| 2021 | 21,480 | 21,270 | 24,240 | 66,990 |
| 2022 | 22,410 | 22,865 | 24,530 | 69,805 |
| 2023 | 23,340 | 24,460 | 24,820 | 72,620 |
| 2024 | 24,270 | 26,055 | 25,110 | 75,435 |
| 2025 | 25,200 | 27,650 | 25,400 | 78,250 |
| 2026 | 26,130 | 29,245 | 25,690 | 81,065 |
| 2027 | 27,060 | 30,840 | 25,980 | 83,880 |
| 2028 | 27,990 | 32,435 | 26,270 | 86,695 |
| 2029 | 28,920 | 34,030 | 26,560 | 89,510 |
| 2030 | 29,850 | 35,625 | 26,850 | 92,325 |
| 2031 | 30,780 | 37,220 | 27,140 | 95,140 |
| 2032 | 31,710 | 38,815 | 27,430 | 97,955 |
| 2033 | 32,640 | 40,410 | 27,720 | 100,770 |
| 2034 | 33,570 | 42,005 | 28,010 | 103,585 |
| 2035 | 34,500 | 43,600 | 28,300 | 106,400 |
| 2036 | 35,190 | 44,472 | 28,866 | 108,528 |
| 2037 | 35,894 | 45,361 | 29,443 | 110,699 |
| 2038 | 36,612 | 46,269 | 30,032 | 112,913 |
| 2039 | 37,344 | 47,194 | 30,633 | 115,171 |
| 2040 | 38,091 | 48,138 | 31,245 | 117,474 |
| 2041 | 38,853 | 49,101 | 31,870 | 119,824 |
| 2042 | 39,630 | 50,083 | 32,508 | 122,220 |
| 2043 | 40,422 | 51,084 | 33,158 | 124,665 |
| 2044 | 41,231 | 52,106 | 33,821 | 127,158 |
| 2045 | 42,055 | 53,148 | 34,498 | 129,701 |
| 2046 | 42,896 | 54,211 | 35,187 | 132,295 |
| 2047 | 43,754 | 55,295 | 35,891 | 134,941 |
| 2048 | 44,629 | 56,401 | 36,609 | 137,640 |
| 2049 | 45,522 | 57,529 | 37,341 | 140,393 |
| 2050 | 46,432 | 58,680 | 38,088 | 143,200 |
| 2051 | 47,361 | 59,853 | 38,850 | 146,064 |
| 2052 | 48,308 | 61,051 | 39,627 | 148,986 |
| 2053 | 49,274 | 62,272 | 40,419 | 151,965 |
| 2054 | 50,260 | 63,517 | 41,228 | 155,005 |

Table 4: Daily Transactions and Revenue for Medium Growth Scenario

|  | Daily Transactions |  |  |  | Daily Revenue |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | BHW_North | BHW_Mid | BHW_South | TOTAL | BHW_North | BHW_Mid | BHW_South | TOTAL |
| 2015 | 14,000 | 3,800 | 15,000 | 32,800 | $3,000 | $1,900 | $4,000 | $8,900 |
| 2016 | 14,885 | 4,110 | 15,220 | 34,215 | 3,710 | 3,180 | 4,465 | 11,355 |
| 2017 | 15,770 | 4,420 | 15,440 | 35,630 | 4,420 | 4,460 | 4,930 | 13,810 |
| 2018 | 16,655 | 4,730 | 15,660 | 37,045 | 5,130 | 5,740 | 5,395 | 16,265 |
| 2019 | 17,540 | 5,040 | 15,880 | 38,460 | 5,840 | 7,020 | 5,860 | 18,720 |
| 2020 | 18,425 | 5,350 | 16,100 | 39,875 | 6,550 | 8,300 | 6,325 | 21,175 |
| 2021 | 19,310 | 5,660 | 16,320 | 41,290 | 7,260 | 9,580 | 6,790 | 23,630 |
| 2022 | 20,195 | 5,970 | 16,540 | 42,705 | 7,970 | 10,860 | 7,255 | 26,085 |
| 2023 | 21,080 | 6,280 | 16,760 | 44,120 | 8,680 | 12,140 | 7,720 | 28,540 |
| 2024 | 21,965 | 6,590 | 16,980 | 45,535 | 9,390 | 13,420 | 8,185 | 30,995 |
| 2025 | 22,850 | 6,900 | 17,200 | 46,950 | 10,100 | 14,700 | 8,650 | 33,450 |
| 2026 | 23,735 | 7,210 | 17,420 | 48,365 | 10,810 | 15,980 | 9,115 | 35,905 |
| 2027 | 24,620 | 7,520 | 17,640 | 49,780 | 11,520 | 17,260 | 9,580 | 38,360 |
| 2028 | 25,505 | 7,830 | 17,860 | 51,195 | 12,230 | 18,540 | 10,045 | 40,815 |
| 2029 | 26,390 | 8,140 | 18,080 | 52,610 | 12,940 | 19,820 | 10,510 | 43,270 |
| 2030 | 27,275 | 8,450 | 18,300 | 54,025 | 13,650 | 21,100 | 10,975 | 45,725 |
| 2031 | 28,160 | 8,760 | 18,520 | 55,440 | 14,360 | 22,380 | 11,440 | 48,180 |
| 2032 | 29,045 | 9,070 | 18,740 | 56,855 | 15,070 | 23,660 | 11,905 | 50,635 |
| 2033 | 29,930 | 9,380 | 18,960 | 58,270 | 15,780 | 24,940 | 12,370 | 53,090 |
| 2034 | 30,815 | 9,690 | 19,180 | 59,685 | 16,490 | 26,220 | 12,835 | 55,545 |
| 2035 | 31,700 | 10,000 | 19,400 | 61,100 | 17,200 | 27,500 | 13,300 | 58,000 |
| 2036 | 32,334 | 10,200 | 19,788 | 62,322 | 17,733 | 28,353 | 13,712 | 59,798 |
| 2037 | 32,981 | 10,404 | 20,184 | 63,568 | 18,283 | 29,231 | 14,137 | 61,652 |
| 2038 | 33,640 | 10,612 | 20,587 | 64,840 | 18,850 | 30,138 | 14,576 | 63,563 |
| 2039 | 34,313 | 10,824 | 20,999 | 66,137 | 19,434 | 31,072 | 15,027 | 65,533 |
| 2040 | 34,999 | 11,041 | 21,419 | 67,459 | 20,036 | 32,035 | 15,493 | 67,565 |
| 2041 | 35,699 | 11,262 | 21,848 | 68,809 | 20,658 | 33,028 | 15,974 | 69,659 |
| 2042 | 36,413 | 11,487 | 22,285 | 70,185 | 21,298 | 34,052 | 16,469 | 71,819 |
| 2043 | 37,142 | 11,717 | 22,730 | 71,588 | 21,958 | 35,108 | 16,979 | 74,045 |
| 2044 | 37,884 | 11,951 | 23,185 | 73,020 | 22,639 | 36,196 | 17,506 | 76,341 |
| 2045 | 38,642 | 12,190 | 23,648 | 74,481 | 23,341 | 37,318 | 18,048 | 78,707 |
| 2046 | 39,415 | 12,434 | 24,121 | 75,970 | 24,064 | 38,475 | 18,608 | 81,147 |
| 2047 | 40,203 | 12,682 | 24,604 | 77,490 | 24,810 | 39,668 | 19,185 | 83,663 |
| 2048 | 41,007 | 12,936 | 25,096 | 79,039 | 25,579 | 40,897 | 19,779 | 86,256 |
| 2049 | 41,827 | 13,195 | 25,598 | 80,620 | 26,372 | 42,165 | 20,393 | 88,930 |
| 2050 | 42,664 | 13,459 | 26,110 | 82,233 | 27,190 | 43,472 | 21,025 | 91,687 |
| 2051 | 43,517 | 13,728 | 26,632 | 83,877 | 28,033 | 44,820 | 21,677 | 94,529 |
| 2052 | 44,388 | 14,002 | 27,165 | 85,555 | 28,902 | 46,209 | 22,349 | 97,460 |
| 2053 | 45,275 | 14,282 | 27,708 | 87,266 | 29,798 | 47,642 | 23,041 | 100,481 |
| 2054 | 46,181 | 14,568 | 28,262 | 89,011 | 30,722 | 49,119 | 23,756 | 103,596 |

High Growth Sensitivity Test

The optimal toll rate per mile in the high growth scenario was $0.12 per mile for all sections of BHW in 2015 and $1.00, $1.00 and $0.60 per mile for the north, middle and south sections respectively for year 2035 (2035 dollars). The corresponding daily transactions and revenue are shown in Table 5. The revenues shown are in future year dollars.

Table 5: Daily Transactions and Revenue for High Growth Scenario

|  | Daily Transactions |  |  |  | Daily Revenue |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | BHW_North | BHW_Mid | BHW_South | TOTAL | BHW_North | BHW_Mid | BHW_South | TOTAL |
| 2015 | 13,400 | 4,200 | 13,600 | 31,200 | $3,600 | $2,700 | $4,600 | $10,900 |
| 2016 | 14,690 | 5,000 | 14,380 | 34,070 | 5,505 | 5,670 | 5,410 | 16,585 |
| 2017 | 15,980 | 5,800 | 15,160 | 36,940 | 7,410 | 8,640 | 6,220 | 22,270 |
| 2018 | 17,270 | 6,600 | 15,940 | 39,810 | 9,315 | 11,610 | 7,030 | 27,955 |
| 2019 | 18,560 | 7,400 | 16,720 | 42,680 | 11,220 | 14,580 | 7,840 | 33,640 |
| 2020 | 19,850 | 8,200 | 17,500 | 45,550 | 13,125 | 17,550 | 8,650 | 39,325 |
| 2021 | 21,140 | 9,000 | 18,280 | 48,420 | 15,030 | 20,520 | 9,460 | 45,010 |
| 2022 | 22,430 | 9,800 | 19,060 | 51,290 | 16,935 | 23,490 | 10,270 | 50,695 |
| 2023 | 23,720 | 10,600 | 19,840 | 54,160 | 18,840 | 26,460 | 11,080 | 56,380 |
| 2024 | 25,010 | 11,400 | 20,620 | 57,030 | 20,745 | 29,430 | 11,890 | 62,065 |
| 2025 | 26,300 | 12,200 | 21,400 | 59,900 | 22,650 | 32,400 | 12,700 | 67,750 |
| 2026 | 27,590 | 13,000 | 22,180 | 62,770 | 24,555 | 35,370 | 13,510 | 73,435 |
| 2027 | 28,880 | 13,800 | 22,960 | 65,640 | 26,460 | 38,340 | 14,320 | 79,120 |
| 2028 | 30,170 | 14,600 | 23,740 | 68,510 | 28,365 | 41,310 | 15,130 | 84,805 |
| 2029 | 31,460 | 15,400 | 24,520 | 71,380 | 30,270 | 44,280 | 15,940 | 90,490 |
| 2030 | 32,750 | 16,200 | 25,300 | 74,250 | 32,175 | 47,250 | 16,750 | 96,175 |
| 2031 | 34,040 | 17,000 | 26,080 | 77,120 | 34,080 | 50,220 | 17,560 | 101,860 |
| 2032 | 35,330 | 17,800 | 26,860 | 79,990 | 35,985 | 53,190 | 18,370 | 107,545 |
| 2033 | 36,620 | 18,600 | 27,640 | 82,860 | 37,890 | 56,160 | 19,180 | 113,230 |
| 2034 | 37,910 | 19,400 | 28,420 | 85,730 | 39,795 | 59,130 | 19,990 | 118,915 |
| 2035 | 39,200 | 20,200 | 29,200 | 88,600 | 41,700 | 62,100 | 20,800 | 124,600 |
| 2036 | 39,984 | 20,604 | 29,784 | 90,372 | 43,368 | 64,584 | 21,632 | 129,584 |
| 2037 | 40,784 | 21,016 | 30,380 | 92,179 | 45,103 | 67,167 | 22,497 | 134,767 |
| 2038 | 41,599 | 21,436 | 30,987 | 94,023 | 46,907 | 69,854 | 23,397 | 140,158 |
| 2039 | 42,431 | 21,865 | 31,607 | 95,903 | 48,783 | 72,648 | 24,333 | 145,764 |
| 2040 | 43,280 | 22,302 | 32,239 | 97,822 | 50,734 | 75,554 | 25,306 | 151,595 |
| 2041 | 44,146 | 22,748 | 32,884 | 99,778 | 52,764 | 78,576 | 26,319 | 157,659 |
| 2042 | 45,028 | 23,203 | 33,542 | 101,774 | 54,874 | 81,719 | 27,371 | 163,965 |
| 2043 | 45,929 | 23,668 | 34,212 | 103,809 | 57,069 | 84,988 | 28,466 | 170,524 |
| 2044 | 46,848 | 24,141 | 34,897 | 105,885 | 59,352 | 88,388 | 29,605 | 177,345 |
| 2045 | 47,785 | 24,624 | 35,595 | 108,003 | 61,726 | 91,923 | 30,789 | 184,438 |
| 2046 | 48,740 | 25,116 | 36,307 | 110,163 | 64,195 | 95,600 | 32,021 | 191,816 |
| 2047 | 49,715 | 25,618 | 37,033 | 112,366 | 66,763 | 99,424 | 33,301 | 199,489 |
| 2048 | 50,709 | 26,131 | 37,773 | 114,614 | 69,434 | 103,401 | 34,634 | 207,468 |
| 2049 | 51,724 | 26,653 | 38,529 | 116,906 | 72,211 | 107,537 | 36,019 | 215,767 |
| 2050 | 52,758 | 27,187 | 39,299 | 119,244 | 75,099 | 111,839 | 37,460 | 224,398 |
| 2051 | 53,813 | 27,730 | 40,085 | 121,629 | 78,103 | 116,312 | 38,958 | 233,373 |
| 2052 | 54,889 | 28,285 | 40,887 | 124,061 | 81,227 | 120,965 | 40,516 | 242,708 |
| 2053 | 55,987 | 28,851 | 41,705 | 126,543 | 84,477 | 125,803 | 42,137 | 252,417 |
| 2054 | 57,107 | 29,428 | 42,539 | 129,073 | 87,856 | 130,835 | 43,822 | 262,513 |

Low Growth Sensitivity Test

The optimal toll rate in the low growth scenario was $0.10 per mile (2015$) for all sections in 2015 and $0.17, $0.35, $0.22 per mile (2035$) in 2035. Table 6 shows the daily transactions and revenue (in future year dollars).

Table 6: Daily Transactions and Revenue for Low Growth Scenario

|  | Daily Transactions |  |  |  | Daily Revenue |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | BHW_North | BHW_Mid | BHW_South | TOTAL | BHW_North | BHW_Mid | BHW_South | TOTAL |
| 2015 | 12,500 | 2,600 | 14,200 | 29,300 | $2,600 | $1,300 | $3,800 | $7,700 |
| 2016 | 13,210 | 2,845 | 14,395 | 30,450 | 2,805 | 1,935 | 3,965 | 8,705 |
| 2017 | 13,920 | 3,090 | 14,590 | 31,600 | 3,010 | 2,570 | 4,130 | 9,710 |
| 2018 | 14,630 | 3,335 | 14,785 | 32,750 | 3,215 | 3,205 | 4,295 | 10,715 |
| 2019 | 15,340 | 3,580 | 14,980 | 33,900 | 3,420 | 3,840 | 4,460 | 11,720 |
| 2020 | 16,050 | 3,825 | 15,175 | 35,050 | 3,625 | 4,475 | 4,625 | 12,725 |
| 2021 | 16,760 | 4,070 | 15,370 | 36,200 | 3,830 | 5,110 | 4,790 | 13,730 |
| 2022 | 17,470 | 4,315 | 15,565 | 37,350 | 4,035 | 5,745 | 4,955 | 14,735 |
| 2023 | 18,180 | 4,560 | 15,760 | 38,500 | 4,240 | 6,380 | 5,120 | 15,740 |
| 2024 | 18,890 | 4,805 | 15,955 | 39,650 | 4,445 | 7,015 | 5,285 | 16,745 |
| 2025 | 19,600 | 5,050 | 16,150 | 40,800 | 4,650 | 7,650 | 5,450 | 17,750 |
| 2026 | 20,310 | 5,295 | 16,345 | 41,950 | 4,855 | 8,285 | 5,615 | 18,755 |
| 2027 | 21,020 | 5,540 | 16,540 | 43,100 | 5,060 | 8,920 | 5,780 | 19,760 |
| 2028 | 21,730 | 5,785 | 16,735 | 44,250 | 5,265 | 9,555 | 5,945 | 20,765 |
| 2029 | 22,440 | 6,030 | 16,930 | 45,400 | 5,470 | 10,190 | 6,110 | 21,770 |
| 2030 | 23,150 | 6,275 | 17,125 | 46,550 | 5,675 | 10,825 | 6,275 | 22,775 |
| 2031 | 23,860 | 6,520 | 17,320 | 47,700 | 5,880 | 11,460 | 6,440 | 23,780 |
| 2032 | 24,570 | 6,765 | 17,515 | 48,850 | 6,085 | 12,095 | 6,605 | 24,785 |
| 2033 | 25,280 | 7,010 | 17,710 | 50,000 | 6,290 | 12,730 | 6,770 | 25,790 |
| 2034 | 25,990 | 7,255 | 17,905 | 51,150 | 6,495 | 13,365 | 6,935 | 26,795 |
| 2035 | 26,700 | 7,500 | 18,100 | 52,300 | 6,700 | 14,000 | 7,100 | 27,800 |
| 2036 | 27,234 | 7,650 | 18,462 | 53,346 | 6,908 | 14,434 | 7,320 | 28,662 |
| 2037 | 27,779 | 7,803 | 18,831 | 54,413 | 7,122 | 14,881 | 7,547 | 29,550 |
| 2038 | 28,334 | 7,959 | 19,208 | 55,501 | 7,343 | 15,343 | 7,781 | 30,466 |
| 2039 | 28,901 | 8,118 | 19,592 | 56,611 | 7,570 | 15,818 | 8,022 | 31,411 |
| 2040 | 29,479 | 8,281 | 19,984 | 57,743 | 7,805 | 16,309 | 8,271 | 32,385 |
| 2041 | 30,069 | 8,446 | 20,384 | 58,898 | 8,047 | 16,814 | 8,527 | 33,388 |
| 2042 | 30,670 | 8,615 | 20,791 | 60,076 | 8,296 | 17,336 | 8,792 | 34,424 |
| 2043 | 31,283 | 8,787 | 21,207 | 61,278 | 8,554 | 17,873 | 9,064 | 35,491 |
| 2044 | 31,909 | 8,963 | 21,631 | 62,503 | 8,819 | 18,427 | 9,345 | 36,591 |
| 2045 | 32,547 | 9,142 | 22,064 | 63,753 | 9,092 | 18,998 | 9,635 | 37,725 |
| 2046 | 33,198 | 9,325 | 22,505 | 65,028 | 9,374 | 19,587 | 9,934 | 38,895 |
| 2047 | 33,862 | 9,512 | 22,955 | 66,329 | 9,664 | 20,194 | 10,241 | 40,100 |
| 2048 | 34,539 | 9,702 | 23,414 | 67,656 | 9,964 | 20,820 | 10,559 | 41,344 |
| 2049 | 35,230 | 9,896 | 23,883 | 69,009 | 10,273 | 21,466 | 10,886 | 42,625 |
| 2050 | 35,935 | 10,094 | 24,360 | 70,389 | 10,591 | 22,131 | 11,224 | 43,947 |
| 2051 | 36,653 | 10,296 | 24,847 | 71,797 | 10,920 | 22,817 | 11,572 | 45,309 |
| 2052 | 37,386 | 10,502 | 25,344 | 73,233 | 11,258 | 23,525 | 11,930 | 46,713 |
| 2053 | 38,134 | 10,712 | 25,851 | 74,697 | 11,607 | 24,254 | 12,300 | 48,162 |
| 2054 | 38,897 | 10,926 | 26,368 | 76,191 | 11,967 | 25,006 | 12,682 | 49,655 |

Gross Annual Revenue

The gross revenue was calculated by applying an annualization factor of 290 days to the total daily revenue for all three sections of BHW combined. Table 7 shows the gross annual revenue for the medium growth scenario in future year dollars (not accounting for unpaid transactions or operations and maintenance costs).

Table 7: Gross Annual Revenue (in future year dollars)

| Year | Medium Growth | High Growth | Low Growth |
| --- | --- | --- | --- |
| 2015 | $2,581,000 | $3,161,000 | $2,233,000 |
| 2016 | 3,292,950 | 4,809,650 | 2,524,450 |
| 2017 | 4,004,900 | 6,458,300 | 2,815,900 |
| 2018 | 4,716,850 | 8,106,950 | 3,107,350 |
| 2019 | 5,428,800 | 9,755,600 | 3,398,800 |
| 2020 | 6,140,750 | 11,404,250 | 3,690,250 |
| 2021 | 6,852,700 | 13,052,900 | 3,981,700 |
| 2022 | 7,564,650 | 14,701,550 | 4,273,150 |
| 2023 | 8,276,600 | 16,350,200 | 4,564,600 |
| 2024 | 8,988,550 | 17,998,850 | 4,856,050 |
| 2025 | 9,700,500 | 19,647,500 | 5,147,500 |
| 2026 | 10,412,450 | 21,296,150 | 5,438,950 |
| 2027 | 11,124,400 | 22,944,800 | 5,730,400 |
| 2028 | 11,836,350 | 24,593,450 | 6,021,850 |
| 2029 | 12,548,300 | 26,242,100 | 6,313,300 |
| 2030 | 13,260,250 | 27,890,750 | 6,604,750 |
| 2031 | 13,972,200 | 29,539,400 | 6,896,200 |
| 2032 | 14,684,150 | 31,188,050 | 7,187,650 |
| 2033 | 15,396,100 | 32,836,700 | 7,479,100 |
| 2034 | 16,108,050 | 34,485,350 | 7,770,550 |
| 2035 | 16,820,000 | 36,134,000 | 8,062,000 |
| 2036 | 17,341,420 | 37,579,360 | 8,311,922 |
| 2037 | 17,879,004 | 39,082,534 | 8,569,592 |
| 2038 | 18,433,253 | 40,645,836 | 8,835,249 |
| 2039 | 19,004,684 | 42,271,669 | 9,109,142 |
| 2040 | 19,593,829 | 43,962,536 | 9,391,525 |
| 2041 | 20,201,238 | 45,721,037 | 9,682,662 |
| 2042 | 20,827,476 | 47,549,879 | 9,982,825 |
| 2043 | 21,473,128 | 49,451,874 | 10,292,292 |
| 2044 | 22,138,795 | 51,429,949 | 10,611,353 |
| 2045 | 22,825,098 | 53,487,147 | 10,940,305 |
| 2046 | 23,532,676 | 55,626,633 | 11,279,455 |
| 2047 | 24,262,189 | 57,851,698 | 11,629,118 |
| 2048 | 25,014,316 | 60,165,766 | 11,989,621 |
| 2049 | 25,789,760 | 62,572,397 | 12,361,299 |
| 2050 | 26,589,243 | 65,075,293 | 12,744,499 |
| 2051 | 27,413,509 | 67,678,304 | 13,139,579 |
| 2052 | 28,263,328 | 70,385,437 | 13,546,906 |
| 2053 | 29,139,491 | 73,200,854 | 13,966,860 |
| 2054 | 30,042,816 | 76,128,888 | 14,399,832 |

O&M Costs

Table 8 shows the total annual Operations and Maintenance (O&M) costs for the medium growth scenarios in year 2012 dollars. Table 8 also illustrates the assumed percentage of unpaid transactions. Several assumptions were used to develop the O&M costs, which include fixed and variable costs (including toll administration costs), and roadway replacement and rehabilitation costs.

Table 8: Annual O&M Costs and Percentage of Unpaid Transactions

| Year | Toll Ops | General Ops | Routine Maint | Major Maintenance | Total O&M O&M | % Unpaid Transactions |
| --- | --- | --- | --- | --- | --- | --- |
| 2015 | $3,200,000 | $4,200,000 | $600,000 | $300,000 | $8,300,000 | 24.9% |
| 2016 | 4,100,000 | 4,200,000 | 600,000 | 600,000 | 9,500,000 | 24.4% |
| 2017 | 4,700,000 | 4,200,000 | 600,000 | 300,000 | 9,800,000 | 24.0% |
| 2018 | 5,400,000 | 4,200,000 | 600,000 | 600,000 | 10,800,000 | 23.6% |
| 2019 | 5,800,000 | 4,200,000 | 600,000 | 700,000 | 11,300,000 | 23.1% |
| 2020 | 5,900,000 | 4,200,000 | 600,000 | 600,000 | 11,300,000 | 22.7% |
| 2021 | 6,100,000 | 4,200,000 | 600,000 | 300,000 | 11,200,000 | 22.3% |
| 2022 | 6,200,000 | 4,200,000 | 600,000 | 600,000 | 11,600,000 | 21.9% |
| 2023 | 6,300,000 | 4,200,000 | 600,000 | 300,000 | 11,400,000 | 21.5% |
| 2024 | 6,400,000 | 4,200,000 | 600,000 | 9,100,000 | 20,300,000 | 21.1% |
| 2025 | 6,500,000 | 4,200,000 | 600,000 | 300,000 | 11,600,000 | 20.7% |
| 2026 | 6,600,000 | 4,200,000 | 600,000 | 14,300,000 | 25,700,000 | 20.3% |
| 2027 | 6,600,000 | 4,200,000 | 600,000 | 300,000 | 11,700,000 | 19.8% |
| 2028 | 6,700,000 | 4,200,000 | 600,000 | 600,000 | 12,100,000 | 19.4% |
| 2029 | 6,800,000 | 4,200,000 | 600,000 | 4,500,000 | 16,100,000 | 19.0% |
| 2030 | 6,800,000 | 4,200,000 | 600,000 | 600,000 | 12,200,000 | 18.6% |
| 2031 | 6,900,000 | 4,200,000 | 600,000 | 300,000 | 12,000,000 | 18.2% |
| 2032 | 7,000,000 | 4,200,000 | 600,000 | 600,000 | 12,400,000 | 17.8% |
| 2033 | 7,000,000 | 4,200,000 | 600,000 | 300,000 | 12,100,000 | 17.4% |
| 2034 | 7,000,000 | 4,200,000 | 600,000 | 9,100,000 | 20,900,000 | 16.9% |
| 2035 | 7,100,000 | 4,200,000 | 600,000 | 300,000 | 12,200,000 | 16.5% |
| 2036 | 7,100,000 | 4,200,000 | 600,000 | 600,000 | 12,500,000 | 16.1% |
| 2037 | 7,100,000 | 4,200,000 | 600,000 | 300,000 | 12,200,000 | 15.7% |
| 2038 | 7,100,000 | 4,200,000 | 600,000 | 14,300,000 | 26,200,000 | 15.3% |
| 2039 | 7,000,000 | 4,200,000 | 600,000 | 4,500,000 | 16,300,000 | 14.9% |
| 2040 | 7,000,000 | 4,200,000 | 600,000 | 600,000 | 12,400,000 | 14.5% |
| 2041 | 7,000,000 | 4,200,000 | 600,000 | 300,000 | 12,100,000 | 14.1% |
| 2042 | 7,000,000 | 4,200,000 | 600,000 | 600,000 | 12,400,000 | 13.6% |
| 2043 | 7,000,000 | 4,200,000 | 600,000 | 300,000 | 12,100,000 | 13.2% |
| 2044 | 7,000,000 | 4,200,000 | 600,000 | 9,100,000 | 20,900,000 | 12.8% |
| 2045 | 6,900,000 | 4,200,000 | 600,000 | 300,000 | 12,000,000 | 12.4% |
| 2046 | 6,900,000 | 4,200,000 | 600,000 | 600,000 | 12,300,000 | 12.0% |
| 2047 | 6,900,000 | 4,200,000 | 600,000 | 300,000 | 12,000,000 | 11.6% |
| 2048 | 6,800,000 | 4,200,000 | 600,000 | 600,000 | 12,200,000 | 11.2% |
| 2049 | 6,800,000 | 4,200,000 | 600,000 | 4,500,000 | 16,100,000 | 10.7% |
| 2050 | 6,700,000 | 4,200,000 | 600,000 | 2,900,000 | 14,400,000 | 10.3% |
| 2051 | 6,700,000 | 4,200,000 | 600,000 | 300,000 | 11,800,000 | 9.9% |
| 2052 | 6,600,000 | 4,200,000 | 600,000 | 600,000 | 12,000,000 | 9.5% |
| 2053 | 6,600,000 | 4,200,000 | 600,000 | 300,000 | 11,700,000 | 9.1% |
| 2054 | 6,500,000 | 4,200,000 | 600,000 | 20,200,000 | 31,500,000 | 8.7% |

Net Annual Revenue

Table 9 shows the total net annual revenue (in future year dollars) for BHW for the medium growth scenario. The net revenue represents the revenue remaining after accounting for unpaid transactions and O&M costs.

Table 9: Net Annual Revenue Accounting for Unpaid Transactions and O&M Costs (in Year 2012 Dollars)

| Year | Gross Annual Revenue (No losses) before O&M (Future Yr $) | Unpaid Annual Transaction (Non-Revenue Generating) Future Yr $ | Gross Annual Revenue Lost due to unpaid transactions before O&M (Future Yr $) | Net Revenue Accounting for Unpaid and O&M Costs (Future Yr $) |
| --- | --- | --- | --- | --- |
| 2015 | $2,581,000 | $641,800 | $1,939,000 | $(6,361,000) |
| 2016 | 3,292,950 | 802,900 | 2,490,000 | (7,010,000) |
| 2017 | 4,004,900 | 960,000 | 3,045,000 | (6,755,000) |
| 2018 | 4,716,850 | 1,111,100 | 3,606,000 | (7,194,000) |
| 2019 | 5,428,800 | 1,256,400 | 4,172,000 | (7,128,000) |
| 2020 | 6,140,750 | 1,395,800 | 4,745,000 | (6,555,000) |
| 2021 | 6,852,700 | 1,529,300 | 5,323,000 | (5,877,000) |
| 2022 | 7,564,650 | 1,656,900 | 5,908,000 | (5,692,000) |
| 2023 | 8,276,600 | 1,778,700 | 6,498,000 | (4,902,000) |
| 2024 | 8,988,550 | 1,894,500 | 7,094,000 | (13,206,000) |
| 2025 | 9,700,500 | 2,004,500 | 7,696,000 | (3,904,000) |
| 2026 | 10,412,450 | 2,108,600 | 8,304,000 | (17,396,000) |
| 2027 | 11,124,400 | 2,206,800 | 8,918,000 | (2,782,000) |
| 2028 | 11,836,350 | 2,299,100 | 9,537,000 | (2,563,000) |
| 2029 | 12,548,300 | 2,385,500 | 10,163,000 | (5,937,000) |
| 2030 | 13,260,250 | 2,466,100 | 10,794,000 | (1,406,000) |
| 2031 | 13,972,200 | 2,540,700 | 11,432,000 | (568,000) |
| 2032 | 14,684,150 | 2,609,500 | 12,075,000 | (325,000) |
| 2033 | 15,396,100 | 2,672,400 | 12,724,000 | 624,000 |
| 2034 | 16,108,050 | 2,729,400 | 13,379,000 | (7,521,000) |
| 2035 | 16,820,000 | 2,780,500 | 14,040,000 | 1,840,000 |
| 2036 | 17,341,420 | 2,795,100 | 14,546,000 | 2,046,000 |
| 2037 | 17,879,004 | 2,807,800 | 15,071,000 | 2,871,000 |
| 2038 | 18,433,253 | 2,818,700 | 15,615,000 | (10,585,000) |
| 2039 | 19,004,684 | 2,827,500 | 16,177,000 | (123,000) |
| 2040 | 19,593,829 | 2,834,200 | 16,760,000 | 4,360,000 |
| 2041 | 20,201,238 | 2,838,600 | 17,363,000 | 5,263,000 |
| 2042 | 20,827,476 | 2,840,500 | 17,987,000 | 5,587,000 |
| 2043 | 21,473,128 | 2,839,800 | 18,633,000 | 6,533,000 |
| 2044 | 22,138,795 | 2,836,400 | 19,302,000 | (1,598,000) |
| 2045 | 22,825,098 | 2,829,900 | 19,995,000 | 7,995,000 |
| 2046 | 23,532,676 | 2,820,400 | 20,712,000 | 8,412,000 |
| 2047 | 24,262,189 | 2,807,600 | 21,455,000 | 9,455,000 |
| 2048 | 25,014,316 | 2,791,200 | 22,223,000 | 10,023,000 |
| 2049 | 25,789,760 | 2,771,200 | 23,019,000 | 6,919,000 |
| 2050 | 26,589,243 | 2,747,200 | 23,842,000 | 9,442,000 |
| 2051 | 27,413,509 | 2,719,100 | 24,694,000 | 12,894,000 |
| 2052 | 28,263,328 | 2,686,600 | 25,577,000 | 13,577,000 |
| 2053 | 29,139,491 | 2,649,400 | 26,490,000 | 14,790,000 |
| 2054 | 30,042,816 | 641,800 | 1,939,000 | (6,361,000) |

Limitations of Analysis

Several broad assumptions were made to complete this analysis:

- Demographic data for the enhanced Mission model are based on models provided by the El Paso MPO

- The network assumptions and model parameters for base and future year are consistent with the information available to HNTB at this time.

- No new trip distribution or trip redistribution was done assuming BHW is included in the roadway network.

The modeling effort for this study used the enhanced Mission model, because a validated Horizon model was not yet available to HNTB at the time of this study. The Horizon model is anticipated to include all existing, committed and planned projects, updated demographic data, new surveys, updated trip generation, updated trip distribution, and trip assignment parameters and methods.

Therefore, the actual results from the completed Horizon model will show differences from the forecast year 2035 traffic volumes presented in this memorandum.

Recommendations

HNTB recommends that this preliminary traffic and revenue study be followed by more comprehensive traffic and revenue studies. Before considering the project for project financing, an investment-grade traffic and revenue study is typical and recommended. A comprehensive investment-grade traffic and revenue study would typically entail detailed land use analysis of the study area by an independent economist, and new origin-destination surveys and stated preference surveys to determine the willingness to pay tolls among potential users of the system.

HNTB also recommends re-evaluating toll sensitivity and updating toll traffic and revenue estimates once the calibrated Horizon travel demand model and stated preference surveys become available because the growth rates and patterns are anticipated to be different compared to the enhanced Mission model.

Disclaimers

HNTB has adopted the most current professional practices, processes and assumptions to develop the preliminary traffic and revenue forecasts presented in this study. However, for any tolled facility, there could be differences between forecasted results and actual results due to circumstances beyond the purview of this study. The traffic and revenue forecasts in this study are only intended to broadly reflect the overall long-term trend. There could be variations for any specific year due to economic conditions and other factors.

Finally, it should be emphasized that the results presented in this study are to be considered preliminary and all findings may be subject to considerable refinement. The project was not analyzed at a level of detail that is sufficient to be used for project financing. More detailed studies involving independent socio-economic data review, stated preference surveys and travel pattern surveys would be required before the findings are considered sufficient for project financing.

- The results presented in this report present the findings based on models, assumptions and information available as of the date of this report.

- All traffic and gross toll revenue forecasts are subject to future economic and social uncertainty, demographic developments due to short-term and long-term deviations from anticipated or planned growth.

- All planned regional transportation projects have been included as are known and available at this time. HNTB cannot predict with certainty any other construction activities beyond the planned projects from the MPO.

- HNTB is not responsible for the socio-economic or demographic forecasts that drive the traffic, or the 2 percent per year growth rate that TPP has directed HNTB to use for forecasts.

- Any changes in any of the other assumptions used in the analysis could materially affect actual toll transactions and revenue.

- Several assumptions regarding toll operating costs, routine maintenance and rehabilitation and replacement costs were made to estimate the O&M costs and unpaid transactions.