# Best Management Practice Inspection Job Aid: Amphibian and Reptile Exclusion Fence

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Was the Amphibian and Reptile Exclusion Fence (AREF) installed

- in accordance with project phasing based on species, and prior

- to the commencement of any work?	☐Y   ☐N   ☐NA

- Has the AREF been installed along the perimeter of the project in

- accordance with the plans?	☐Y   ☐N   ☐NA

- Is the AREF the outer-most Best Management Practice (BMP)?	☐Y   ☐N   ☐NA

- Are steel posts T-posts with a minimum 4-foot length and

- minimum weight of 1.25 pounds per foot?	☐Y   ☐N   ☐NA

- Are steel posts imbedded a minimum of 18 inches deep or

- adequately anchored, if in rock?	☐Y   ☐N   ☐NA

- Are steel posts spaced at 6-foot intervals?	☐Y   ☐N   ☐NA

- Is geo-textile fabric used in accordance with DMS-6230 with

- width of minimum 3 feet?	☐Y   ☐N   ☐NA

- Is net reinforcement 12.5-gauge minimum standard wire

- gauge (SWG) galvanized wire mesh with maximum opening

- of 2 inches by 4 inches and minimum width of 30 inches?

- OR 12.5-gauge SWG minimum hinge joint knot woven mesh,

- with horizontal and vertical wires spaced maximum

- 12 inches apart?	☐Y   ☐N   ☐NA

- Is net reinforcement attached to steel posts with four evenly

- spaced T-clips or sewn vertical pockets?	☐Y   ☐N   ☐NA

- Is filter fabric secured to net reinforcement using hog rings or

- cord at a maximum spacing of 15 inches?	☐Y   ☐N   ☐NA

- Is filter fabric installed to face away from construction, and net

- reinforcement towards construction?	☐Y   ☐N   ☐NA

- Is filter fabric trenched in 6 inches deep and 6 inches horizontally

- (facing away from construction, and trench backfill has been hand

- tamped?	☐Y   ☐N   ☐NA

- Do filter fabric splices have a minimum of 2-foot overlap and

- located at steel posts?	☐Y   ☐N   ☐NA

- Are splices smooth and taped down to prevent gaps?	☐Y   ☐N   ☐NA

- Are spliced sections outside of areas with concentrated flow?	☐Y   ☐N   ☐NA

- Are J-hooks of minimum 18 inches installed at driveways and

- roadway intersections?	☐Y   ☐N   ☐NA

- Are culverts fully encompassed or excluded on both ends of the

- AREF?	☐Y   ☐N   ☐NA

- Is the AREF marked with spray painted letters in a bright color

- every 50 feet?	☐Y   ☐N   ☐NA

## Inspection criteria

- Are inspections occurring at correct frequency based on species

- specific criteria?	☐Y   ☐N   ☐NA

- Are all site personnel appropriately trained based on species

- specific criteria?	☐Y   ☐N   ☐NA

- Is the 24-inch vertical clearance (from group to top of AREF)

- being maintained on both sides?	☐Y   ☐N   ☐NA

- Is sedimentation, vegetation, and other debris removed daily?	☐Y   ☐N   ☐NA

- Are holes of less than 1 inch diameter being repaired using duct

- tape daily?	☐Y   ☐N   ☐NA

- Is filter fabric replaced for sections of AREF with holes of 1 inch

- diameter or larger are discovered?	☐Y   ☐N   ☐NA

- Has the filter fabric begun to photodegrade?	☐Y   ☐N   ☐NA

- Has the AREF been damaged by construction activities?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Have all required repairs been made in a timely manner?	☐Y   ☐N   ☐NA

- Are all maintenance and repairs being tracked properly?	☐Y   ☐N   ☐NA

- Have the biologists been notified of any major issues

- encountered?	☐Y   ☐N   ☐NA

- Is the only reason maintenance has not occurred due to

- “Too Wet Too Work”?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

## References

Texas Department of Transportation (TxDOT) Amphibian and Reptile Exclusion Fence detail sheet: AREF-21

TxDOT Special Specification 5003 Amphibian and Reptile Exclusion Fence