# Best Management Practice Section and Design Job Aid: Amphibian and Reptile Exclusion Fence

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

The Amphibian and Reptile Exclusion Fence (AREF) is a barrier structure installed on construction projects to prevent amphibians and reptiles from entering the construction site. It is used on projects with threatened or endangered amphibian and reptile species. Check with the Environmental Coordinator for project requirements.

Figure 1. Amphibian and reptile exclusion fencing were installed to help prevent amphibians and reptiles from entering a construction site.

## Types

Standard Amphibian and Reptile Exclusion Fence (AREF) – AREF is used in standard conditions.

Toad Exclusion Fence (TEF) - TEF has special inspection and training requirements associated with it. Include TEF Special Provision.

## Application

AREF is used to prevent amphibians and reptiles from entering the project area:

Along perimeter of project

Around culverts extending into the project area

Along streams and channels

At driveways and connecting roadways

Around temporary stockpiles

Around trenches and excavations

Around materials storage areas

### Advantages

Cost effective way to prevent protected amphibians and reptiles from entering the project area

### Disadvantages

If they are not installed and maintained properly, animals could enter project area

## Design Criteria

Phase installation based on species specific timeframes, such as breeding season.

Place AREF along the perimeter of the project as the outermost Best Management Practice (BMP).

In locations where the AREF abuts driveways, roadways, or other intersections include a j-hook as shown on the detail sheet (AREF-21).

AREF must be designed to either fully encompass or fully exclude both ends of a culvert.

Add any specific AREF requirements to the general notes in the plan set.

Use AREF linestyle when showing the location of the AREF in the plan set.

## References

Texas Department of Transportation (TxDOT) Amphibian and Reptile Exclusion Fence detail sheet: AREF-21

TxDOT Special Specification 5003 Amphibian and Reptile Exclusion Fence