# Best Management Practice Inspection Job Aid: Vertical Tracking

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Is the vertical tracking installed in accordance with the

- Stormwater Discharge Map or Stormwater Pollution Prevention

- Plan (SWP3) Plan Sheets?	☐Y   ☐N   ☐NA

- Does the dozer have a track undercarriage capable of producing

- soil impression of at least 12 inches in length by 2 inches to

- 4 inches in width by 12 inch to 2 inches in depth?	☐Y   ☐N   ☐NA

- Are there more than 12 inches between track impressions?	☐Y   ☐N   ☐NA

- Are linear track impressions perpendicular to the slope or

- direction of water flow?	☐Y   ☐N   ☐NA

- Are liner track impressions continuous?	☐Y   ☐N   ☐NA

## Inspection criteria

- Is vertical tracking functioning properly (for example, reduced

- erosion, no dust)?	☐Y   ☐N   ☐NA

- Has vertical tracking been washed out by heavy rains?	☐Y   ☐N   ☒NA

- Has any erosion been observed on the face of the sloped surface?	☐Y   ☐N   ☐NA

- Is blowing dust observed? If yes, then additional controls are

- needed.	☐Y   ☐N   ☐NA

- Does the sloped area need to be re-tracted?	☐Y   ☐N   ☐NA

- Is vertical tracking functioning appropriately as a standalone

- Best Management Practice (BMP)?	☐Y   ☐N   ☐NA

- Do additional BMPs need to be used in conjunction with

- vertical tracking (for example, seeding, mulching, etcetera)?	☐Y   ☐N   ☐NA

- Were maintenance needs recommended to the contractor?	☐Y   ☐N   ☐NA

- Were any other concerns identified with the vertical tracking?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Is vertical tracking implementation being maintained?	☐Y   ☐N   ☐NA

- Have recommended maintenance activities been conducted

- (for example, slope erosion repair)?	☐Y   ☐N   ☐NA

- Have additional BMPs been implemented for use in conjunction

- with vertical tracking?	☐Y   ☐N   ☐NA

- Do additional BMPs require maintenance?	☐Y   ☐N   ☐NA

- Were any additional maintenance concerns observed and

- shown to the Contractor?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

## Reference

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment and Water Pollution Control Measures: Fence and Vertical Tracking detail sheet: EC (1) – 16