# Best Management Practice Section and Design Job Aid: Vertical Tracking

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Vertical tracking is a temporary soil stabilization technique for bare, uncompacted soil on a slope which uses tracked machinery to create depressions parallel to slope contours on the face of sloped surfaces.

Figure 1. Parallel vertical track marks in soil created by heavy construction equipment.

## Application

Vertical tracking is required on projects where soil disturbing activities on slopes are planned as a temporary erosion control practice. Vertical tracking temporarily stabilizes the soil by:

Reducing runoff velocity

Compacting soil on slopes

Increasing rainfall infiltration

Providing sediment trapping

Helping the establishment of vegetation

### Advantages

Cost effective

Reduces erosion due to wind and water

Temporarily stabilizes soil

### Disadvantages

Effectiveness is only temporary

Tracking can be washed away by heavy rain

Usage limited by the soil type

May be required to have other sediment and soil stabilizing controls

## Design Criteria

- Equipment with track undercarriage capable of producing linear soil impressions should have the following measurements:

Minimum Length: 12 inches

Width: 2 inches to 4 inches

Depth: 12 inch to 2 inches

- Depths of the bare, uncompacted soil and slope of the surface should be considered.

- Continuous linear track impressions should occur where the minimum 12-inch length impressions are perpendicular to the slope or direction of water flow.

- There should be no more than 12 inches between track impressions.

- Entire face of sloped area should have vertical tracking.

- Number of passes on the surface should be limited to avoid compaction of the soil.

- Indicate on the plans where vertical tracking will be required.

- Track impressions must be perpendicular to the direction of runoff.

- Vertical tracking must be used as a temporary erosion control measure for all projects that disturb soil on any slope, unless otherwise approved.

## References

TxDOT Temporary Erosion, Sediment and Water Pollution Control Measures: Fence & Vertical Tracking: EC (1) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 4.4.10)