# Best Management Practice Inspection Job Aid: Floating Turbidity Barrier

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Are the Floating Turbidity Barriers (FTBs) installed in accordance with

- the Stormwater Discharge Map or Stormwater Pollution Prevention

- Plan (SWP3) layout sheets?	☐Y   ☐N   ☐NA

- Were the FTBs installed prior to construction or land

- disturbing activities?	☐Y   ☐N   ☐NA

- Does the top section of the FTB consist of 18 to 22 ounces

- Polyvinyl Chloride (PVC) coated nylon fabric?	☐Y   ☐N   ☐NA

- Is the top section of the FTB painted in a bright orange or yellow

- to increase visibility?	☐Y   ☐N   ☐NA

- Do the FTB specifications consist of a geo-synthetic having a

- filtration Apparent Opening Size (AOS) of 0.220 millimeters (mm)

- maximum for non-woven geotextiles, and AOS of 0.425 mm maximum

- for woven textiles, when tested in accordance with American Society

- for Testing and Materials (ASTM) D 4751-99a?	☐Y   ☐N   ☐NA

- Is the opening of the FTB made of a non-woven material that

- cannot be enlarged under pressure or by being snagged?	☐Y   ☐N   ☐NA

- Do the FTB specifications include a minimum grab strength of

- 300 pounds per square inch (psi) when tested in accordance with

- American Society for Testing and Materials (ASTM) D 4632-91?	☐Y   ☐N   ☐NA

- Does the FTB floatation material have a minimum of 6 inches

- of freeboard?	☐Y   ☐N   ☐NA

- Do the load lines consist of a minimum 516 inch vinyl coated

- galvanized aircraft cable with 9800 pounds breaking strength?	☐Y   ☐N   ☐NA

- Do the mooring and adjustment lines consist of 12 inch nylon

- rope?	☐Y   ☐N   ☐NA

- Does the ballast consist at a minimum of 516 inch galvanized steel

- chain?	☐Y   ☐N   ☐NA

- Are all buoys securely attached?	☐Y   ☐N   ☐NA

- Are lighted buoys installed at 100-foot increments along the

- centerline of the FTB?	☐Y   ☐N   ☐NA

- If there is traffic on the water body (for example, canoes) and

- the FTB impedes the traffic, are there signs indicating where to

- exit the water to portage beyond the FTB?	☐Y   ☐N   ☐NA

## Inspection criteria

- Is the FTB functioning properly (for example, removing sediment

- from water, sediment build up, etcetera)?	☐Y   ☐N   ☐NA

- Is the FTB still secured by lines or other anchors?	☐Y   ☐N   ☐NA

- Has there been any displacement of FTB, such as by wind or

- wave action?	☐Y   ☐N   ☐NA

- Are panel section connections secure?	☐Y   ☐N   ☐NA

- Are all floats intact?	☐Y   ☐N   ☐NA

- Is the filter material clogged with sediment or other debris?	☐Y   ☐N   ☐NA

- Is an abnormal amount of sediment built up along the FTB?	☐Y   ☐N   ☐NA

- Were any additional concerns identified with the FTB?	☐Y   ☐N   ☐NA

- Were maintenance requirements recommended to the contractor?	☐Y   ☐N   ☐NA

- Were any additional concerns identified with the FTB?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Is the FTB in good condition without any visible damage?	☐Y   ☐N   ☐NA

- Are associated Best Management Practices (BMPs) functioning

- properly?	☐Y   ☐N   ☐NA

- Are more BMPs needed to prevent sediment deposition?	☐Y   ☐N   ☐NA

- Has the FTB been damaged by a large storm event?	☐Y   ☐N   ☐NA

- Is the sediment being removed during maintenance being properly

- disposed of or managed?	☐Y   ☐N   ☐NA

- Were all clogs within the FTB addressed, if any?	☐Y   ☐N   ☐NA

- Did the contractor address all recommended maintenance needs?	☐Y   ☐N   ☐NA

- Were any additional BMPs recommended to the Contractor?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

### Reference

Texas Department of Transportation (TxDOT) Special Specification 5002 Floating Turbidity Barrier