# Best Management Practice Section and Design Job Aid: Biodegradable Erosion Control Log

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Biodegradable Erosion Control Logs (ECLs) are long flexible logs comprised of a core material that is biodegradable, or recyclable, wrapped in a biodegradable containment mesh. ECLs intercept sediment from sheet flow adjacent to various structures, can act as velocity dissipation devices, and can serve as a perimeter sediment barrier control.

Figure 1. Biodegradable erosion control logs installed along a roadside shoulder to reduce soil erosion and support vegetation establishment.

## Types and Configurations

Erosion Control Log Dam

Erosion Control Log on Slopes

Erosion Control Log at Back of Curb

Erosion Control Log at Drop Inlet

Erosion Control Log at Edge of Right of Way

Erosion Control Log at Curb Inlet

Erosion Control Log at Cur and Grade Inlet

## Application

Biodegradable Erosion Control Logs are used to slow flow and act as sediment barriers.  Control logs should be used for the following:

Perimeter protection

Slope protection

Stockpile controls

Curb and grade inlet protection

Sediment traps

Sediment barriers

Drainage ditch dam

Dewatering activities

### Advantages

Able to treat sheet flows over short distances

Able to provide perimeter control where other best management practices are not suitable due to soil type or paved surfaces

Easy to install

Can be used on steep slopes

### Disadvantages

Difficult to remove when wet or filled with sediment

Limited drainage area for sediment capture

Additional controls may be needed to remove fine soils from runoff

## Design Criteria

The drainage area for a log sediment trap should not exceed 5 acres.

The trap capacity should be 1800 cubic feet per acre (0.5 inches over the drainage area).

Space logs a minimum of 500 feet on center when used in drainage ditches (dam).

Use logs immediately preceding ditch inlets (inlet protection).

Logs used for inlet protection must not impede vehicle traffic or flood the roadway.

Ensure logs used for slope protection are spaced according to the diameter and slope (See Sheet 2 of 3 in EC (9)-16).

Use logs before drainage leaves the right-of-way or construction site (perimeter control).

Use logs to control sediment before drainage enters a water course.

## References

TxDOT Temporary Erosion, Sediment and Water Pollution Control Measures: Erosion Control Log: EC (9) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.10 & 4.4.9)