# Best Management Practice Inspection Job Aid: Biodegradable Erosion Control Log

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Are the biodegradable control logs (ECLs) installed in accordance

- with the Stormwater Discharge Map or Stormwater Pollution

- Prevention Plan (SWP3) layout sheets?	☐Y   ☐N   ☐NA

- Were the control logs installed prior to construction and or land

- disturbing activities?	☐Y   ☐N   ☐NA

- Have the control logs been filled with sufficient filter material to

- achieve the minimum compacted diameter specified in the plans

- without excessive deformation?	☐Y   ☐N   ☐NA

- Is the core material biodegradable and or recyclable and in

- accordance with Item 506, Section 2.10.1 Core Material?	☐Y   ☐N   ☐NA

- Is the containment mesh 100% biodegradable, photodegradable,

- or recyclable and in accordance with Item 506, Section 2.10.2

- Containment Mesh?	☐Y   ☐N   ☐NA

- Are stakes 2 inches by 2 inches thick wood or #3 rebar, 2 feet

- to 4 feet long, embedded such that 2 inches protrudes above log?	☐Y   ☐N   ☐NA

- Are stakes installed on downhill side of control log as indicated

- by the Erosion Control Sheets?	☐Y   ☐N   ☐NA

- Are compost cradles installed under control log dams and control

- logs at the back of curbs and edge of right of ways?	☐Y   ☐N   ☐NA

- Are control logs on slope installed at least 5 feet above toe of

- slope and 6 feet below toe of slope?	☐Y   ☐N   ☐NA

- Are the joints of the control logs on slope staggered at least

- 5 feet to 10 feet? Are the control logs spaced according to the

- Erosion Control Log Spacing Table?	☐Y   ☐N   ☐NA

- Are the control logs on slope anchored in a trench sized using the

- Trench Depth Table?	☐Y   ☐N   ☐NA

- Are the sandbags used as anchors placed on top of control log

- and sized to hold control logs in place?	☐Y   ☐N   ☐NA

- Are drainage inlets surrounded by the control logs? Are they

- impeding traffic?	☐Y   ☐N   ☐NA

## Inspection criteria

- Are the Best Management Practices (BMPs) functioning

- properly (for example, no flow under logs, sediment buildup,

- etcetera)?	☐Y   ☐N   ☐NA

- Are the control logs still secured by stakes, rebar, or other

- anchorage?	☐Y   ☐N   ☐NA

- Has there been any displacement of control logs?	☐Y   ☐N   ☐NA

- Are additional upstream stakes or rebar needed to prevent

- erosion control log from folding in on itself?	☐Y   ☐N   ☐NA

- Do control logs at curb inlets impede traffic or flood the roadway?	☐Y   ☐N   ☐NA

- Were any concerns identified with the biodegradable erosion

- control logs?	☐Y   ☐N   ☐NA

- Were maintenance requirements recommended to the contractor?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Are control logs in good condition without any visible damage?	☐Y   ☐N   ☐NA

- Are associated BMPs functioning properly?	☐Y   ☐N   ☐NA

- Has sediment in the control log accumulated to a depth of ½ the

- diameter? If yes, then remove sediment.	☐Y   ☐N   ☐NA

- Is the sediment being removed during maintenance and being

- properly disposed of or managed?	☐Y   ☐N   ☐NA

- Did the contractor address all recommended maintenance needs?	☐Y   ☐N   ☐NA

- Were any additional BMPs recommended to the Contractor?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

### References

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment and Water Pollution Control Measures: Erosion Control Log detail sheet: EC (9) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Section 2.10 & 4.4.9)