# Best Management Practice Inspection Job Aid: Sediment Control Fence

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Have the sediment control fence location(s) been documented in

- the Stormwater Pollution Prevention Plan (SWP3) Layout Sheets?	☐Y   ☐N   ☐NA

- Is the sediment control fence installed in accordance with the

- SW3P Layout Sheets?	☐Y   ☐N   ☐NA

- Was the sediment control fence installed prior to construction

- and land disturbing activities?	☐Y   ☐N   ☐NA

- Has the drainage area above the sediment control fence been

- evaluated (for size, slope, concentrated flow, etcetera) to determine

- if Stormwater Control Facility (SCF) is the appropriate Best

- Management Practice (BMP) for the location?	☐Y   ☐N   ☐NA

- Is the welded wire mesh (WWM) or fabric properly connected

- to the posts (in other words, 6 consecutive hog rings for WWM,

- 4 evenly spaced staples or T-Clips)?	☐Y   ☐N   ☐NA

- Have splices in fabric been overlapped at least 6 inches? 	☐Y   ☐N   ☐NA

- Are steel or wood posts at least 4 feet in height, embedded at

- least 6 inches, spaced at 6 inches to 8 inches apart, and angled

- at least 2 inches toward runoff slope?	☐Y   ☐N   ☐NA

- Is the trench size a minimum of 6 inches by 6 inches?	☐Y   ☐N   ☐NA

- Was it verified that 4 inches to 6 inches of fabric were placed

- against the trench side and 2 inches of fabric were placed

- across the trench bottom?	☐Y   ☐N   ☐NA

- Has the trench been backfilled and tamped to prevent

- undercutting of the sediment control fence?	☐Y   ☐N   ☐NA

## Inspection criteria

- Is the BMP operating properly (for example, sediment buildup)?	☐Y   ☐N   ☐NA

- Has erosion occurred around or behind the sediment control

- fence?	☐Y   ☐N   ☐NA

- Are sections of the sediment control fence damaged (for

- example, run over, ripped, undercut)?	☐Y   ☐N   ☐NA

- Are posts still embedded at least 6 inches?  Is the fabric still

- toed in along the entire length of the fence?	☐Y   ☐N   ☐NA

- Does the sediment control fence have adequate sediment

- holding capacity (for example, sediment accumulation less

- than 13 barrier height)?	☐Y   ☐N   ☐NA

- Has the filter fabric begun to photodegrade?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Is the sediment control fence maintained and in good condition?	☐Y   ☐N   ☐NA

- Has sediment been removed to increase holding capacity?	☐Y   ☐N   ☐NA

- Have splits, tears, slumping, or weathered fabric of the sediment

- control fence been repaired?	☐Y   ☐N   ☐NA

- Were maintenance needs and or additional BMPs recommended

- to the Contractor? If yes, list the additional BMPs in Form 2118.	☐Y   ☐N   ☐NA

- Has the contractor addressed all maintenance requirements

- (for example, flooding, sediment buildup, etcetera)?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

## Reference

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment and Water Pollution Control Measures: Fence and Vertical Tracking detail sheet: EC (1) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.9 & 4.4.8)