# Best Management Practice Inspection Job Aid: Stabilized Construction Exit

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Are stabilized construction exit (SCE) locations shown in the Stormwater Pollution Prevention Plan (SWP3)?	☐Y   ☐N   ☐NA

- For Type 1: Is the exit foundation flexible base, bituminous

- concrete, Portland cement concrete, or material approved by the

- Engineer? Is coarse aggregate 4 inches to 8 inches thick?	☐Y   ☐N   ☐NA

- For Type 3 – rock construction: Is the exit foundation flexible

- base, bituminous concrete, Portland cement concrete, or material

- approved by the Engineer? Is coarse aggregate 2 inches to

- 4 inches thick? Is aggregate spread a minimum of 4 inches thick?	☐Y   ☐N   ☐NA

- For Type 2: Are railroad ties and timbers at least No. 2 grade and

- free from large and loose knots? Are timbers fastened with at

- least ½ by 6-inch nuts, bolts, and or lag bolts?	☐Y   ☐N   ☐NA

- For Type 3 – timber construction: Is plywood or pressed water

- board at least ½ inch thick? Are treated timber planks at least

- No. 2 grade and free from large and loose knots? Are 2-inch by

- 4-inch timbers nailed onto ends of wood sheets?	☐Y   ☐N   ☐NA

- For Type 1 or 2: Does the exit extend at a minimum of 50 feet

- from one approach transition to another?	☐Y   ☐N   ☐NA

- For Type 3: Does the length match the length shown on the

- plans?	☐Y   ☐N   ☐NA

- For Type 1 or 2: Is the foundation course at least 6 inches thick?	☐Y   ☐N   ☐NA

- For Type 1 or 2: Is the exit approach transition no steeper

- than 6‑to‑1 (H:V) gradient?	☐Y   ☐N   ☐NA

- For Type 1 or 2: Is the exit and or road graded properly to allow

- drainage to a sediment trapping device?	☐Y   ☐N   ☐NA

- For Type 1 or 2: Is the exit the proper width (in other words,

- 14 feet for one-way traffic, 20 feet for two-way traffic)?	☐Y   ☐N   ☐NA

## Inspection criteria

- Is the SCE functioning properly (for example, no tracking of

- sediments onto public roadways, dust control for section

- roadway, etcetera)?	☐Y   ☐N   ☐NA

- Are any clogs or voids in the aggregate visible?	☐Y   ☐N   ☐NA

- Is sediment visible on the public roadway?	☐Y   ☐N   ☐NA

- Is there a sediment trapping device and is it being

- bypassed?	☐Y   ☐N   ☐NA

- Are timber planks fastened properly?	☐Y   ☐N   ☐NA

- Were any concerns identified with the SCE?	☐Y   ☐N   ☐NA

- Were maintenance requirements recommended to the contractor?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Is the SCE in good shape without any visible damage?	☐Y   ☐N   ☐NA

- Did the contractor address all recommended maintenance needs?	☐Y   ☐N   ☐NA

- Were any additional Best Management Practices (BMPs)

- recommended to the Contractor (for example, dust control, street

- sweeping, wheel washing, etcetera)?	☐Y   ☐N   ☐NA

- Was the SCE properly removed when no longer needed?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

## References

Texas Department of Transportation (TxDOT) Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.4 & 4.4.4)

TxDOT Temporary Erosion, Sediment and Water Pollution Control Measures: Construction Exits: EC (3) – 16