# Best Management Practice Section and Design Job Aid: Street Sweeping

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Street sweeping is the practice of removing tracked sediment from construction sites and from public roads. Removing the tracked sediment prevents it from entering storm drains and water bodies. It also helps to alleviate vehicular safety hazards.

Figure 1. Street sweeping equipment stationed adjacent to a construction project.

## Types

Mechanical sweeper

Vacuum-assisted, dry, waterless sweeper

Vacuum-assisted sweeper

Regenerative-air sweeper

## Application

All tracked sediment originating from the construction site should be removed from public and private roadways via sweeping or vacuuming.  Street sweeping is utilized for sediment control, waste management, and good housekeeping.

### Advantages

Cost effective form of sediment capture

Easily performed during construction activities

The use of centralized entrances and or exits will help minimize tracked soil from leaving the site

### Disadvantages

Difficult to sweep and or vacuum soil that is wet or compact

Street vacuums man not be allowed in cities

## Design Criteria

- Implement street sweeping practices wherever construction exits enter a public roadway.

- Limit egress and ingress locations to minimize tracked soil.

- Clearly indicate where and when street sweeping must be implemented on the site plans or construction notes.

- Used with the implementation of a stabilized construction exit.

## References

Texas Pollutant Discharge Elimination System (TPDES) Construction General Permit TXR150000 Part III.F and Part IV.

Texas Department of Transportation (TxDOT) Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.4 & 4.4.4)