# Best Management Practice Section and Design Job Aid: Rock Filter Dam

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Rock filter dams are small barriers consisting of various sized aggregate that are used to reduce the velocities of concentrated flows, provide a sediment barrier, and reduce erosion in channels, swales, dikes, or ditches.

Figure 1. Rock filter dam installed in a vegetated drainage swale.

## Types

Type 1–18 inches high with no wire mesh and 3 inches to 6 inches aggregate

Type 2–18 inches high with wire mesh and 3 inches to 6 inches aggregate

Type 3–36 inches high with wire mesh and 4 inches to 8 inches aggregate

Type 4–Sack gabions with 3 inches to 6 inches aggregate

Type 5–Use rock filter dams as shown on plans

## Application

Rock Filter Dams should be constructed upstream and or downstream from disturbed areas to decrease stormwater flow velocity, allowing sediment to settle out, in concentrated overland flow. Typical locations for Rock Filter Dams include the following:

At toe of slope

At sediment trap

At channel sections

In ditches, dikes, and swale outlets

In stream flow

### Advantages

Reduces erosive velocities

Effective at removing some sediments

Can be used with other channel protection controls

### Disadvantages

Maintenance required after heavy or high velocity flows

## Design Criteria

Place Rock Filter Dam near the toe of slopes where erosion is anticipated, upstream and downstream from drainage structures, and in roadway ditches and channels.

Dimensions must be indicated on the Stormwater Pollution Prevention Plan.

The dams should be sized to filter a maximum flow through rate of 60 gallons per minute per square foot of cross-sectional area.

A 2-year storm frequency may be used to calculate flow rate.

Rock filter dams should be placed perpendicular to the flow of the stream or channel, unless otherwise directed.

Side slopes should be 2‑to‑1 (H:V) gradient or flatter. Dams within the safety clear zone shall have side slopes of 6‑to‑1 (H:V) gradient or flatter.

Type 1 should be used at toe of slope, around inlets, in small ditches, and at dike and swale outlets.

Type 1 may not be used in concentrated high velocity flows (approximately 8 feet per second or more).

Type 2 should be used in ditches and dike or swale outlets.

Type 3 should be used in stream flow (anchor to stream bed).

Type 4 should be used in ditches and small channels.

## References

TxDOT Temporary Erosion and Water Pollution Control Measures: Rock Filter Dams: EC (2)-16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.1 & 4.4.1)