# Best Management Practice Inspection Job Aid: Rock Filer Dam

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Have the rock filter dams been documented on the Stormwater

- Discharge Map or Stormwater Pollution Prevention Plan (SWP3)

- Plan Sheets?	☐Y   ☐N   ☐NA

- Are the rock filter dams installed in accordance with the

- Stormwater Discharge Map or SWP3 Plan Sheets?	☐Y   ☐N   ☐NA

- Were the rock filter dams installed prior to construction and

- or land disturbing activities?	☐Y   ☐N   ☐NA

- Are the rock filter dam dimensions as indicated on the

- SW3P plans?	☐Y   ☐N   ☐NA

- Are the side slopes for all dam types 2‑to‑1 (H:V) gradient or

- flatter? Do all dam types in safety (clear) zones have side slopes

- of 6‑to‑1 (H:V) gradient or flatter?	☐Y   ☐N   ☐NA

- Is this rock filter dam at a sediment trap? If yes, refer

- to EC (2)-16 “V” Shape Plan View for side slope information.	☐Y   ☐N   ☐NA

- For rock filter dams types 1, 2, and 3, is the top width at

- least 2 feet?	☐Y   ☐N   ☐NA

- Are rock filter dams being secured a minimum of 4 inches into

- existing ground?	☐Y   ☐N   ☐NA

- For type 1 rock filter dams, are sandbags used at embedded

- foundation to increase filter efficiency of low flows?	☐Y   ☐N   ☐NA

- Is there at least 1 foot between top of rock filter dam weir and

- top of embankment for filter dams at sediment traps?	☐Y   ☐N   ☐NA

- Are type 2 and 3 rock filter dams secured with 20 gauge

- galvanized woven wire mesh with 1 inch diameter hexagonal

- openings?	☐Y   ☐N   ☐NA

- For type 2 and 3 rock filter dams in streams, was galvanized

- mesh secured or staked to stream bed prior to aggregate

- placement?	☐Y   ☐N   ☐NA

- Are sack gabions staked down with 34 inch diameter rebar

- stakes, and have a double-twisted hexagonal weave with a

- nominal mesh opening of 212 inch by 314 inch?	☐Y   ☐N   ☐NA

- Is each sack gabion 3 feet, 6 feet, or 9 feet long? Does each

- sack gabion have a 2-foot diameter?	☐Y   ☐N   ☐NA

- Is flow outlet onto a stabilized area (for example, vegetation,

- rock, etcetera)?	☐Y   ☐N   ☐NA

- Are materials used in accordance with the specifications for

- “Rock Filter Dams for Erosion and Sedimentation Control”

- (for example, aggregate, wire mesh, sandbags, etcetera)?	☐Y   ☐N   ☐NA

## Inspection criteria

- Are the Best Management Practices (BMPs) functioning properly?	☐Y   ☐N   ☐NA

- Has any of the aggregate been washed out of the filter dam?	☐Y   ☐N   ☐NA

- Is there any erosion along the edges of the dam where it

- meets the side of ditch, swale, or channel?	☐Y   ☐N   ☐NA

- For dam types 2, 3, and 4, is there any clogging in the wire mesh?	☐Y   ☐N   ☐NA

- For dam types 2, 3, and 4, is there any damage to the wire mesh?	☐Y   ☐N   ☐NA

- Were any concerns identified with the rock filter dams?	☐Y   ☐N   ☐NA

- Were maintenance requirements recommended by the contractor?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Are rock filter dams in good condition without any visible damage?	☐Y   ☐N   ☐NA

- Are associated BMPs functioning properly (for example, reduced

- velocity, sediment build-up)?	☐Y   ☐N   ☐NA

- Has sediment in the rock filter dam accumulated around the dam?

- If yes, remove.	☐Y   ☐N   ☐NA

- Did the contractor address all recommended maintenance needs?	☐Y   ☐N   ☐NA

- Were any additional BMPs recommended to the Contractor?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

### Reference

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment, and Water Pollution Control Measures: Rock Filter Dams: EC (2) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls (Sections 2.1 & 4.4.1)