# Best Management Practice Section and Design Job Aid: Outlet Protection

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Outlet Protection is a temporary erosion control and functions by dispersing concentrated flow and slowing flow velocity at drainage pipe outlets, the outlet end of an armored flume or swale, and other points where concentrated flow is discharged to an open channel.

Figure 1. Outlet protection constructed of crushed rock surrounded by wire mesh installed at a culvert discharge to stabilize the embankment and prevent erosion.

## Types

Crushed rock

Gabions

Manufactured velocity dissipators

Gabion drop structures

Rock riprap

Articulated concrete blocks

Temporary baffled chutes

Stabilized grade breaks

## Application

Outlet protection devices are used to protect outfalls from scouring from concentrated flows by stabilizing the soil, protecting channels receiving concentrated flow, and providing permanent stabilization.

### Advantages

Protects habitat

Protects new conveyance systems from damage due to erosion until permanent controls are installed

May be reusable in some instances

### Disadvantages

Can increase erosion

May not be as cost effective as permanent structures

Can be damaged by large storm events

## Design Criteria

Outlet protection should not cause flooding or block flow of stormwater.

Outlet protection should be installed on all outlets where the design storm velocity exceeds 4 feet per second and discharges to an unlined or natural channel.

Install outlet protection at the following locations:

- Pipe, conduit, drain, culvert, and diversion dike outlets; concentrated flow outlets to streams; outlet to larger channel; and areas where soil is susceptible to scour.

Permanent dissipation devices should be utilized early in construction, when possible, to eliminate need for temporary devices.

If riprap is utilized, the apron should be aligned with flow direction. The length of the apron should be 4.5 times the outlet pipe diameter and the width should be 4.0 times the outlet pipe diameter.

Riprap cannot be used at outlets discharging to channels with different elevations.

Design calculations should be completed for any outlet protection and should include:

- Discharge velocity: stone size; and apron dimension.

## References

Texas Pollutant Discharge Elimination System (TPDES) Construction General Permit TXR150000 Part III Section F

TxDOT Temporary Erosion, Sediment and Water Pollution Control Measures Temporary Pave Flumes: EC (8) - 16