# Best Management Practice Section and Design Job Aid: General Material and Equipment Management

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

General material and equipment management addresses the practices associated with proper storage, maintenance, and use of general site materials and equipment at the construction site.

Figure 1. Designated storage container for materials and equipment illustrating proper material management practices that reduce weather exposure and support site safety and compliance.

## Application

General material and equipment management is applicable to all construction sites. The objective of this Best Management Practice (BMP) is to prevent the improper storage and use of material and equipment at the construction site. General material and equipment include, but are not limited to, the following:

Machinery

Oils

Antifreeze

Pesticides

Herbicides

Fertilizers

Paints

Solvents

Wash Water

Safety materials

Adhesives

Asphalt byproducts

Concrete byproducts

### Advantages

Keeps construction site clean and orderly

Helps prevent improper use

Works in conjunction with other housekeeping BMPs

### Disadvantages

Does not address spill and leak response procedures or chemical management

Can be costly to maintain

## Design Criteria

All construction sites must implement the general material and equipment management BMP.

Clearly identify or add notes to plan sheets to indicate where general construction material and equipment must be stored.

The contractor should designate site personnel responsible for implementing general material and equipment management.

All site personnel must be informed of the proper storage and use of materials.

All general storage and equipment areas should be located at least 50 feet away from inlets, channels, and surface waters.

Equipment should be stored orderly and according to manufacturer specifications.

Store materials indoors in existing structures if possible, to reduce contact with stormwater.

Evaluate the site for the need for temporary storage sheds or buildings.

All material and equipment on site should be cataloged to determine storage and maintenance needs.

This BMP must be used in conjunction with the following BMPs:

Vehicle and equipment management

Street sweeping

Chemical management

Contractor waste management

Material and equipment use over water

Sanitary waste management

Spill and leak control

Stockpile management

## References

Texas Pollutant Discharge Elimination System (TPDES) Construction General Permit TXR 150000 Part III Section F and Part IV