# Best Management Practice Section and Design Job Aid: Temporary Pipe Slope Drain

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Temporary Pipe Slope Drain (PSD) is a temporary pipeline that drains concentrated surface runoff safely down slopes to prevent slope erosion.

Figure 1. Illustration of a temporary pipe slope drain installed on a slope.

## Application

Temporary PSDs are used to protect long, un-stabilized sloped areas from erosion caused by overland flow. Applicable site conditions where PSDs are used include, but are not limited to, the following:

Large Berms

Steeply graded and or disturbed soil

Significant grade changes

Roadway embankments

Levees

Stream banks (site specific)

### Advantages

Protects slopes from overland flow erosion

Safe way to convey water down steep slopes

Capable of controlling runoff from drainage areas greater than 5 acres

### Disadvantages

Drain does not treat the runoff – treatment through a sediment control is required before runoff is discharged offsite

Drain can easily be damaged by construction traffic

Difficult to secure pipe to the slope

Clogs easily during large rain events

May require grading upstream to direct water to the pipe inlet

## Design Criteria

- The drainage area contributing runoff to a PSD should not exceed 5 acres.

- The top of an embankment or earth dike should be formed over the top of the inlet pipe that has a slope of 3% or greater.

- The outlet pipe shall have a slope of 1% or less and discharge into an energy dissipator or sediment trap.

- Additional BMPs must be included to assist with directing water into the pipe (interceptor swales, diversion dikes, etcetera).

- Space must be provided for an energy dissipating outlet to be installed.

- The PSD should be sized to drain the peak rate of runoff without overtopping the earth dike entrance. A 25-year storm frequency may be used to calculate the flow rate.  (Additional sizing criteria should be in accordance with the PSD Design Criteria table).

## References

Texas Department of Transportation (TxDOT) Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, September 1, 2024, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls

TxDOT Temporary Erosion, Sediment and Water Pollution Control Measures: Pipe Slope Drain: EC (7) – 16