District Control Plan Sheet Checklist for Submittal Review

# NOTE: EDIT THIS CHECKLIST TO MEET DISTRICT NEEDS. AN EDITABLE VERSION IS AVAILABLE FROM THE ROW DIVISION SURVEY SECTION ON REQUEST FROM DISTRICT SURVEY COORDINATORS.

| TxDOTCONNECT ROW Project No.: |  |  |  |
| --- | --- | --- | --- |
| RCSJ: |  |  | Highway: |
| CCSJ: |  |  | County: |
| Limits: |  |  |  |
| Date Reviewed: |  |  | By: |

## General Requirements

- All map sheets are legible: no light, dark, faded, or overlapping text

- All maps plotted on 11”x17” map sheets with half-inch borders

- All map borders are a minimum of ½” with 1 ½” margin on left

- Text size on all plots must be at least .06 inch (Engineering Font)

- Graphic files are in current format; text files are in Microsoft Word

- Control meets the TxDOT’s level of accuracy

## Control Index Sheet Requirements

- Table containing primary survey control monument information include:

  - Control Point Name

  - Northing and Easting coordinates

  - Latitude and Longitude (at the State Project Manager or District Survey Coordinator’s discretion)

  - Elevation

  - Station and Offset (at the State Project Manager or District Survey Coordinator’s discretion)

  - Description of monument (eg. 5/8” REBAR WITH 3 ¼” ALUMINUM CAP IN CONCRETE stamped “TEXAS DEPT OF TRANSPORTATION CONTROL MARK”) Monument Specifications

  - Monuments described for size, material and stamp. Be specific.

- Layout shows side streets with street names

- Control points:

  - Use “” symbol for primary control points

  - Use “” symbol for secondary control points

  - Use “” symbol for panel points

  - Label control point names near symbols

  - Show offsite control points in relation to project

- Layout shows all county lines and city limit lines with labels

- Line work from planimetric (optional)

- All coordinates have commas and two decimal places

- All latitudes and longitudes are in DMS and have four decimal places

- Insets showing reference ties to each survey control monument (optional)

- North arrow shown in upper right corner

- Layout scale shown or N.T.S. (under North arrow)

- Legend of Symbols and Conventional Signs shown in lower left corner (location optional)

- Project baseline – Includes:

  - Baseline bearings and distances or curve data

  - Baseline equations (as applicable)

  - Label 500-foot stations

  - Label PC’s and PT’s along baseline

- Begin Project and End Project Include:

  - Stationing

  - CSJ number

  - TXDOTCONNECT Project Number

  - North and East coordinates with commas and two decimal places

  - Latitude and Longitude are in DMS with four decimal places

- Title block (lower right corner):

  - Firm’s name, TBPELS Firm Registration number, address, and phone number

  - County

  - Highway designation

  - CSJ

  - TXDOTCONNECT Project Number

  - Federal Road Division Number is 6

  - State District Number

  - Federal Aid Project Number is blank

  - Sheet number #

  - Most current TxDOT Logo placed in the title block (available in the ORD Workspace)

  - Copyright “© month/year” (latest amended date)

  - TEXAS DEPARTMENT OF TRANSPORTATION       DISTRICT

  - State District Address and Phone Number

- Signature and seal of Registered Professional Land Surveyor (RPLS) with date (above Revision area)

- Printed surveyors name and RPLS number under seal

- Revision area above title block is clear of text and graphics (2”x2” minimum)

- Revision month/year and description of revision shown for control sheet only (if applicable): most recent revision listed on top

- Metadata/Notes included:

  - The basis of bearing shall be “Grid North”

  - Texas Coordinate System, State Plane Zone. (specify zone used)

  - North American Datum of 1983 (NAD 83), 2011 Adjustment, 2010.0 Epoch

  - North American Vertical Datum 88 (NAVD 88) GEOID______ (eg. GEOID18) (if applicable)

  - Indicate surface or grid coordinates and the agreed upon surface adjustment factor for the project. (Surface Coord. = Grid Coord. x S.A.F. of      )

  - Unit of Measure (US Survey Feet)

  - Date of survey (last month/day/year of survey field work)

  - Indicate how the horizontal and vertical control was established

  - Indicate which TxDOT RRP Station used. If static was done indicate which three TxDOT RRP Stations used.

- Check old redlines for correction (if applicable)

## Primary Control Data Sheet Requirements

Note: Secondary Control data sheets may be added at the State Project Manager or District Survey Coordinator’s discretion. The secondary control data sheets shall follow the same criteria as the primary.

- Title block (lower right corner) (See Section B 14)

- Control Monument Sketch:

- Maximum of nine sketches per sheet

- Show Reference Ties (minimum of three per sketch) &  label objects being tied

- North arrow pointing North

- Scale used or N.T.S.

- Name of adjacent road(s)

- Monument described for size, material and stamp. Be specific.

- Monument name shown next to monument symbol

- Use “” for primary control points

- N and E coordinates (with commas and two decimal places)

- Elevation shown to 0.01 foot

- Ties shown to nearest foot

- Location Description (along bottom edge of cell) – Brief text describing the general location of the control point in relation to major intersections, and which side of the road the control point can be found.

- Baseline shown and labeled with stationing, PC’s and PT’s along baseline

- All coordinates have commas and two decimal places.

- Any labeled latitudes and longitudes are in DMS with four decimal places. (eg. 01°02’03.1234”)

- Metadata/Notes: (in upper right margin)

- The basis of bearing shall be “Grid North”.

- Texas Coordinate System, State Plane Zone (specify zone used)

- North American Datum of 1983 (NAD 83), 2011 Adjustment, 2010.0 Epoch

- North American Vertical Datum 88 (NAVD 88) GEOID______ (eg. GEOID18) (if applicable)

- Indicate surface or grid coordinates and the agreed upon surface adjustment factor for the project (Surface Coord = Grid Coord x S.A.F. of ________________)

- Unit of Measure (US Survey Feet)

- Date of survey (last month/day/year of survey field work)

- Indicate how the horizontal and vertical control was established

- Indicate which TxDOT RRP Station used. If static was done indicate which three TxDOT RRP Stations used.

- Legend under Basis of bearing and coordinates information. Legend should include line style, control monuments, signs etc.

- Signature and seal of Registered Professional Land Surveyor with date (above Revision area)

- Printed surveyors name and RPLS number under seal

- Revision area above title block is clear of text and graphics (2”x2” minimum)

- Revision month/year and description of revision shown for control sheet only (if applicable): most recent revision listed on top

- No ownership information shown on Horizontal and Vertical Control Data Sheet

- Check old redlines for correction (if applicable)

- Secondary Control and Inverse Data Sheet Requirements

Note: The Secondary Control and Inverse data may be placed on the Control Index sheet at the State Project Manager or District Survey Coordinator’s discretion.

- Title block (lower right corner) (see section B 14)

- Basis of bearings and coordinates: (in upper right margin)

- Texas Coordinate System, State Plane Zone (specify zone used)

- North American Datum of 1983 (NAD 83), 2011 Adjustment Epoch 2010.00

- North American Vertical Datum 88 (NAVD 88) GEOID______ (eg. GEOID18) (if applicable)

- US Survey Feet

- Indicate surface to grid combined adjustment factor

- Indicate how the horizontal and vertical control was established

- Indicate which TxDOT RRP Station used. If static was done indicate which three TxDOT RRP Stations used.

- Date of Survey

- Secondary Control point and or Panel Point Table include:

- Point Number

- Northing

- Easting

- Elevation

- Descriptions of Monument

- Survey Control Inverse Table include: (only intervisible monuments)

- From Point Number

- To Point Number

- Bearing (eg. N 01° 02’ 03” E)

- Distance (eg. 123.45’)

- Elevation Difference (eg. 123.45’)

- Signature and seal of Registered Professional Land Surveyor with date (above Revision area)

- Printed surveyors name and RPLS number under seal

- Revision area above title block is clear of text and graphics (2”x2” minimum)

- Revision month/year and description of revision shown for control sheet only (if applicable): most recent revision listed on top.

## Deliverables

- General

- Control Index Sheet

- Primary Control Data Sheet

- Secondary Control and or Panel Point Table Sheet

- Most recent Form ROW-S-2462 for each primary control monument

- Field Data

- Field Notes/Sketches/Level Loop

- Photos

- ASCII or Text file of all control monuments recovered; Surface and Grid (Point, Northing, Easting, Elevation, Code)

- ASCII or Text file of all control monuments to be set; Surface and Grid (Point, Northing, Easting, Elevation, Code)

- Office Data

- TxDOT design software files (project, points database, chains, input & output files). see the Design Survey Checklist

- RAW files (conventional), GPS observation files, photos, static, network adjustment, digital level loop or other measurement files

Control Plan Sheet Checklist completed by:

| Name: | Date: |
| --- | --- |

Additional notes: